| Course Name: | Microprocessors and Peripherals (2UXC404) | Semester: | IV |
| --- | --- | --- | --- |
| Date of Performance: | 06 / 05 / 2021 | Batch No: | B2 |
| Faculty Name: | Prof. Kirti Sawlani | Roll No: | 1912052 |
| Faculty Sign & Date: |  | Grade/Marks: | ___/25 |

Miniproject

Title: Traffic Lights Simulator

Aim and Objective of the Experiment:

Aim: Developing a Traffic Light system using the inbuilt trafficlight.exe in 8086 Objectives: Ensuring smooth flow of traffic in simulator by the use of traffic lights and delays. .

COs to be achieved:

CO 2. Develop 8086 based assembly language programs for various applications.

Useful links

Simulator/Emulator: https://emu8086-microprocessor-emulator.en.softonic.com/download DOSBox x86 emulator https://sourceforge.net/projects/dosbox/

Work done

- Write report of your miniproject here containing abstract, introduction, code and screenshots of your output.

Roll no. 1912052 – Vedant Kelkar 1912060 – Nachiket Naik

There are 7 devices attached to the emulator: traffic lights, stepper-motor, LED display, thermometer, printer, robot and simple test device. We can view devices when you click "Virtual Devices" menu of the emulator.

Code:

#start=Traffic_Lights.exe# name "traffic2" data segment stop_all dw 0000001001001001b;0249h s0 dw 0000001100001100b;030Ch s1 dw 0000010010010010b;0492h s2 dw 0000100001100001b;0861h s3 dw 0000010010010010b;0492h endofday = $ data ends code segment assume sc:code, ds:data, ss:stack start:mov ax,data mov ds,ax ;starting all red mov ax, stop_all out 4, ax ;situation vertical movement next: mov si, offset s0 mov ax, [si] out 4, ax ;delay mov bx,0fh repeat:mov cx,5fH b:loop b dec bx jnz repeat ;situation yellow mov si,offset s1 mov ax, [si] out 4, ax ;delay mov bx,0fh reps:mov cx,1fH bac:loop bac dec bx jnz reps ;situation horizontal movement mov si,offset s2 mov ax, [si] out 4, ax ;delay mov bx,0fh repet:mov cx,5fH bk:loop bk dec bx jnz repet ;situation yellow mov si,offset s3 mov ax, [si] out 4, ax ;delay mov bx,0fh r:mov cx,1fH a:loop a dec bx jnz r cmp si, endofday jz next mov si, offset s0 jmp next code ends end start

The code will run for infinite time as the outer loop’s condition will never be satisfied and the program will never break out of the loop.

Conclusion:

Thus, we have written a machine code program which controls a set of traffic lights and provides the timing of the light sequence with the emu8086 emulator.

Signature of faculty in-charge with Date: