| Course Name: | Microprocessors and Peripherals (2UXC404) | Semester: | IV |
| --- | --- | --- | --- |
| Date of Performance: | 17-02-2021 | Batch No: | B2 |
| Faculty Name: | KCS | Roll No: | 1912052 |
| Faculty Sign & Date: |  | Grade/Marks: | ___/25 |

Experiment No: 3

Title: Multiplication of 32 bit numbers

Aim and Objective of the Experiment:

Aim: Write an 8086 based ALP to - Multiply two 32 bit numbers stored in the data segment and store the result back in the data segment. Objectives: To study basic instructions and addressing modes of 8086. Understand assembler directives and concept of data and code segment This experiment covers following instructions groups. - Data transfer - Arithmetic ( Multiply instructions)

COs to be achieved:

CO 2. Develop 8086 based assembly language programs for various applications.

Useful links

NASM Assembler https://www.tutorialspoint.com/compile_assembly_online.php MASM/TASM Assembler

Work to be done

- Upload image of handwritten algorithm/flowchart and lst file of the program and output screenshots . Also upload results for post lab questions. data segment n1h dw 1234h n1l dw 5678h n2h dw 1234h n2l dw 5678h prod dw 4 dup(0) data ends code segment assume cs: code,ds:data start: mov ax,data mov ds,ax mov cx,0 mov ax,n1l mul n2l mov prod,ax mov prod+2,dx mov ax,n1h mul n2l add prod+2,ax adc prod+4,dx mov ax,n2h mul n1l add prod+2,ax adc prod+4,dx jnc it4 inc cx it4: mov ax,n1h mul n2h add prod+4,ax adc dx,cx add prod+6,dx mov ah,4ch int 21h code ends end start HW data segment n1h dw 1234h n1l dw 1234h n2h dw 5678h n2l dw 5678h prod dw 4 dup(0) data ends code segment assume cs: code,ds:data start: mov ax,data mov ds,ax mov cx,0 mov ax,n1l mul n2l mov prod,ax mov prod+2,dx mov ax,n1h mul n2l add prod+2,ax adc prod+4,dx mov ax,n2h mul n1l add prod+2,ax adc prod+4,dx jnc it4 inc cx it4: mov ax,n1h mul n2h add prod+4,ax adc dx,cx add prod+6,dx mov ah,4ch int 21h code ends end start

Post Lab Subjective/Objective type Questions:

Q.1 Write an 8086 based ALP to find the factorial of a number in data segment and store the result back in data segment data segment A db 05h fact dw 4 dup(0) data ends code segment assume cs: code,ds:data start: mov ax,data mov ds,ax mov ah,00 mov al,A X:dec A mov cl,A cmp cl,01 jz stop mul A jmp X stop: mov fact,ax mov ah,4ch int 21h code ends end start Q.2. What is the output of the following instruction? AX = 37D7H, BH = 151 decimal DIV BH Remainder will be stored in AH Quotient in AL AH=65H=10 decimal AL=5EH=94 decimal Q.3 What is the difference between MUL and IMUL? Explain with example MUL Multipl byte or word (unsigned) MUL,Integer multiply byte or word (signed)

Conclusion:

Wrote 8086 program to multiply two 32 bit numbers stored in the data segment and store the result back in the data segment.

Signature of faculty in-charge with Date: